FOSTER ON LINE

NARRATION SCRIPT

MODULE:	6

| SLIDE | NARRATION |
| --- | --- |
| 1 | IN MODULE SIX, YOU WILL BEGIN TO UNDERSTAND THE ROLE PACKAGING PLAYS IN PROTECTING YOU AND THE PUBLIC FROM THE HAZARDS OF RADIOLOGICAL MATERIAL. |
| 2 | AS A RESPONDER, YOU SHOULD BE AWARE THAT RAM COMES IN PACKAGES OF ALL SIZES AND SHAPES. YOU WILL SEE SOME EXAMPLES OF THIS SHORTLY. THESE PACKAGES CAN BE ANY WEIGHT, BUT GENERALLY, RAM PACKAGING IS MUCH HEAVIER THAN THAT USED FOR OTHER MATERIALS. RAM IS ALSO SHIPPED VIA ALL MODES OF TRANSPORT. MOST COMMONLY IT IS SHIPPED BY TRUCK, BUT IT IS ALSO SHIPPED BY AIR, WATER, AND RAIL. FINALLY, AND IMPORTANTLY, UNLIKE OTHER PACKAGING, ANY RADIOACTIVE MATERIAL THAT COULD POTENTIALLY HARM YOU OR THE ENVIRONMENT MUST BE SHIPPED IN PACKAGING THAT IS CONSTRUCTED TO MEET STRINGENT FEDERAL STANDARDS. |
| 3 | THE LEAST HAZARDOUS CLASS OF PACKAGING IS FOR WHAT IS CALLED EXCEPTED MATERIAL. THESE ARE MATERIALS THAT HAVE VERY LIMITED AMOUNTS OF RADIOACTIVE MATERIAL, AND THE MATERIAL IS INCORPORATED INTO A SPECIFIC ITEM. PACKAGING FOR THESE MATERIALS IS NOT SUBJECT TO ANY SPECIAL FEDERAL DESIGN REQUIREMENTS. THE EXCEPTED PACKAGE IS NOT REQUIRED TO HAVE A RADIOACTIVE LABEL. THESE MATERIALS ARE SHIPPED VIA COMMON CARRIER, AND ARE NOT CLASSIFIED AS HAZARDOUS CARGO. SOME EXAMPLES OF EXCEPTED MATERIALS INCLUDE SMOKE DETECTORS, SELF-ILLUMINATED EXIT SIGNS, AND TRITIUM GUN SIGHTS. |
| 4 | THE NEXT LEVEL IS DESIGNATED AS TYPE A PACKAGING. MATERIALS SHIPPED IN TYPE A PACKAGING ARE SMALL AMOUNTS OF RADIOACTIVE MATERIAL. TYPE A PACKAGES MUST MEET FEDERAL REQUIREMENTS. |
| 5 | TYPE A PACKAGES MUST UNDERGO A SERIES OF TESTS BEFORE BEING USED TO TRANSPORT RADIOLOGICAL MATERIAL. THESE TESTS INCLUDE: BEING SUBJECTED TO A WATER SPRAY THAT SIMULATES A 2 INCH PER HOUR RAINFALL FOR AT LEAST ONE HOUR BEING DROPPED ONTO A HARD, FLAT SURFACE HAVING A WEIGHT AT LEAST 5 TIMES THE WEIGHT OF THE PACKAGE STACKED ON TOP OF IT FOR AT LEAST 24 HOURS, AND HAVING A ONE AND A QUARTER INCH, 13 POUND STEEL BAR DROPPED ONTO IT FROM A DISTANCE OF 3.3 FEET ABOVE THE PACKAGE. ONLY IF THE PACKAGE PASSES ALL OF THESE TESTS WITHOUT ALLOWING THE CONTENTS TO BE DAMAGED IS IT APPROVED FOR USE IN SHIPPING. |
| 6 | THIS ARE EXAMPLES OF TYPE A PACKAGING. AS YOU CAN SEE, THE PACKAGES ARE QUITE DIFFERENT IN SIZE, SHAPE, CONSTRUCTION, AND USE. |
| 7 | THIS IS A TYPICAL TYPE A PACKAGE YOU MIGHT ENCOUNTER IN A TRANSPORTATION ACCIDENT. IT RESEMBLES AN ORDINARY CARDBOARD BOX FROM THE OUTSIDE. INSIDE, YOU CAN SEE HOW THE PACKAGING IS ARRANGED. THERE IS A DIVIDER TO HOLD STYROFOAM POPCORN AROUND THE CENTRAL CHAMBER. INSIDE THE CENTER IS THE RADIOACTIVE MATERIAL WHICH IS INSIDE A LEAD CONTAINER CALLED A PIG. THIS PIG IS, IN TURN, PLACED INSIDE A SEALED CAN. LOOKING CLOSER AT THE BOX, THERE ARE SOME ADDITIONAL REQUIRED LABELS. FIRST IS THE PRINTED SPECIFICATION THAT THIS IS A TYPE A PACKAGE. SECOND ARE THE TWO ARROWS POINTING UP. THIS NOT ONLY SHOWS THE CORRECT ORIENTATION FOR THE PACKAGE, BUT INDICATES THAT THE CONTENTS ARE A LIQUID. FINALLY, THE DESCRIPTION OF THE CONTENTS, INCLUDING THE U N NUMBER ARE PRINTED ON THE OUTSIDE. AS YOU CAN SEE, EVEN THIS SEEMINGLY SIMPLE CARDBOARD BOX IS ACTUALLY A WELL DESIGNED SYSTEM WHICH PROVIDES A LOT OF PROTECTION FOR THE CONTENTS. |
| 8 | SHOWN HERE IS A COMPLETELY DIFFERENT TYPE A PACKAGE. THIS LOOKS LIKE A CARRYING CASE, WITH A YELLOW 2 LABEL. AS WITH THE BOX, THIS CASE ALSO HAS THE TYPE A DESIGNATION PRINTED ON IT, AND HAS THE ORIENTATION ARROWS. INSIDE, THIS CASE HAS SEVERAL LEAD CONTAINERS PROTECTED BY CUSHIONING. |
| 9 | THE MOST COMMON TYPE A MATERIALS ARE RADIOPHARMACEUTICALS USED IN MEDICAL TESTING AND TREATMENT. SOME RADIOPHARMACEUTICALS INCLUDE RADIOIODINE, IRIDIUM, TECHNETIUM, AND GALLIUM. |
| 10 | IT IS IMPORTANT FOR YOU TO UNDERSTAND… ALL TYPE A PACKAGES WILL HAVE “TYPE A” PRINTED ON THE PACKAGE, AND WILL HAVE A RADIOACTIVE LABEL APPLIED. TYPE A CONTAINERS ARE DESIGNED TO WITHSTAND ORDINARY SHIPPING AND HANDLING CONDITIONS. MOST IMPORTANTLY, TYPE A PACKAGES WILL NEVER CONTAIN LIFE ENDANGERING QUANTITIES OF RADIOACTIVE MATERIAL. |
| 11 | FOR HIGHER LEVELS OF RADIOLOGICAL MATERIAL, THE TYPE B CONTAINER MUST BE USED. TYPE B PACKAGES ARE TIGHTER, STRONGER, AND HEAVIER THAN TYPE A. AND AS SHOWN IN THESE PICTURES, TYPE B CONTAINERS ALSO COME IN A VARIETY OF SIZES AND SHAPES. |
| 12 | THESE ARE ALSO TYPE B PACKAGES. YOU MAY SEEN THE ONES IN THE RIGHT HAND PICTURE ON OUR HIGHWAYS. THESE ARE CALLED TRUPAC SHIPPING CASKS, AND ARE USED TO TRANSPORT CONTAMINATED MATERIALS TO A STORAGE FACILITY IN NEW MEXICO. |
| 13 | TYPE B CONTAINERS MUST UNDERGO TESTING FAR MORE DEMANDING THAN TYPE A. TYPE B SHIPPING CONTAINERS MUST WITHSTAND ALL OF THE TYPE A TESTS, PLUS THE FOLLOWING TYPE B TESTS: A DROP OF 30 FEET ONTO A HARD, UNYIELDING SURFACE, STRIKING THE WEAKEST POINT OF THE CONTAINER BEING DROPPED ONTO A STEEL ROD 6 INCHES IN DIAMETER AND 8 INCHES LONG FROM A HEIGHT OF 40 INCHES. BEING EXPOSED TO A FLAMMABLE LIQUID FUELED POOL FIRE WITH A TEMPERATURE OF 1, 475 DEGREES FOR AT LEAST 30 MINUTES. SUBMERGED IN A POOL OF WATER NOT LESS THAN FIFTY FEET DEEP FOR AT LEAST 8 HOURS. AS WITH THE TYPE A PACKAGES, THE TYPE B MUST PASS THESE TESTS WITHOUT ANY LEAKAGE. |
| 14 | IN SOME CASES, THE CONTAINERS ARE TESTED UNDER CONDITIONS THAT ARE FAR BEYOND WHAT MAY BE EXPECTED IN REAL WORLD CONDITIONS, AS SHOWN IN THESE PICTURES. THIS TYPE B CONTAINER WAS CRASHED INTO A 690 TOME CONCRETE BARRIER AT 60 MILE PER HOUR. WHILE THE YELLOW IMPACT CUSHION MATERIAL AND THE CAB OF THE TRUCK WERE BOTH DESTROYED, THE CONTAINER DID NOT FAIL. |
| 15 | IN ANOTHER TEST, A DIFFERENT TYPE OF CONTAINER IS CRASHED INTO THE SAME 690 TON WALL WHILE CARRIED ON A TRAIN CAR MOVING AT 81 MILES PER HOUR. AS WITH THE TRUCK TEST, THE CUSHIONING FEATURES OF THE PACKAGE COUPLED WITH THE STRENGTH OF CONSTRUCTION KEPT THE CONTENTS SAFE. |
| 16 | SOME OTHER THINGS ABOUT TYPE B PACKAGES YOU SHOULD KNOW… TYPE B PACKAGES MUST INCORPORATE A FEATURE THAT SHOWS THE PACKAGE HAS NOT BEEN OPENED BY UNAUTHORIZED PERSONS. MUST BE PERMANENTLY MARKED WITH ITS MODEL NUMBER, SERIAL NUMBER, GROSS WEIGHT, AND PACKAGE IDENTIFICATION NUMBER. MUST BE PERMANENTLY MARKED “TYPE B”. |
| 17 | THIS CONCLUDES MODULE 6. IN MODULE 7, WE WILL SHOW YOU SOME OF THE RADIOLOGICAL MATERIAL YOU MAY ENCOUNTER AS AN EMERGENCY RESPONDER. |